NOAA WP-3D N42RF ERROR SUMMARY

1

HURRICANE LESLIE 2012

8 Sept 2012 0412A LESLIE

HRD (PHX) KFLL KFLL

Flight ID: 20120908H1

Sensor or system						Number or Name

Altitude							AltGPS.1 (RINU)

Accelerometer						AccZfilterI-GPS.1

Dew Point Probe						TDM.2

Dynamic Pressure						PQM.2

Inertial Selected						INE1

Static Pressure						PSM.2

Temperature Probe				                     TTM.2

Constants File                                                           AAMPSConfig/core/n43.xml

Flight Directory						acdata/2012/MET/20120908H1

Local Met Data			Takeoff (0755z)		Landing (1614z)

Aircraft Static Pressure		  1013.8 mb			     1014.7 mb

Tower Pressure (corrected)	  1012.1 mb			     1014.5 mb

__________________________________________________________________

Notes:

The storm was flown at 8,000 feet absolute altitude (to better accommodate the IWRAP).

The Maycomm (TDM.3) dew point sensor repeatedly spiked in areas of heavy precipitation.  TDM.2 (the Edgetech) is the reference.

Vertical Winds during the storm portion of the flight from 11:00z to 13:26z showed a mean UWZ of +0.01 m/s. SPECIAL NOTE: The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm. It is recommended that these values be used for vertical wind analysis.

Beginning at 15:35:29z and continuing until 15:41:43z, 1-second data gaps were observed in several derived parameter values (those with the .d extension). These occurrences were very infrequent and greatly varied among the list of derived parameter values. In other words, wind direction (WD.d) may show a 1-second gap once during the flight but no other derived parameter values would exhibit a data gap for that same time. Likewise after a period of time elapsed a different derived parameter's value, say true airspeed (TAS.d), would be missing but other derived parameter outputs would be present.
Since it would be tedious and time consuming to list all the individual derived parameters values missing for a specific time, it was decided to provide the customer the beginning and end times for the time period when these data gaps occurred.

All other flight level instruments worked optimally during the flight.

- 12 dropsondes deployed, all good and transmitted

- 12 externally loaded AXBT were deployed (2 were bad)

Flight Director: Richard Henning (813) 828-3310 ext. 3086 and Ian Sears