Flight ID: 20121015H1

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.1

Dew Point Probe						TDM.2X

Static Pressure							PSM.2

Dynamic Pressure						PQM.2

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2012/20121015H1

Constants File	              20121015H1/AAMPSConfig/core/n42.xml

Local Met Data:			Takeoff (1809Z)		Landing (0243Z)

Aircraft Static Pressure (PSM.2)	1013.3 mb			1009.6 mb

Tower Pressure (corrected)		1012.1 mb			1011.4 mb

Notes:

The Edgetech dewpoint, TDM.2, performed best and was used as default.   However, it spiked erroneously on descent for landing from 02:32:09Z – 02:36:29Z, when TDM.1 also spiked.  Because of this, the Maycomm TDL was used to substitute during this time as:  TDM.2X = TDM.3 – 1.5.  Dew point values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.

The Novatel Alt, Lat and Lon (GPS.3) had several one second data spikes from 20:16Z to 01:33Z.  The blended inertial-GPS solution Alt, Lat and Lon (I-GPS.1) is the default position source.

Radome variables PQM.4, PTR, PDAlpha.2, PDBeta.2 are all erroneous while the radome was in icing conditions from 19:43Z – 20:13Z and again from 00:16Z – 01:46Z.  PQM.2, PDAlpha and PDBeta # 1 are all default.

SPECIAL NOTE!!!    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds, vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 13 GPS dropsondes (5 sent) and 2 AXBT’s released from the aircraft.  There were 6 hurricane penetrations.