NOAA WP-3D N42RF ERROR SUMMARY

1

HURRICANE SANDY 2012

26 Oct 2012 0918A SANDY

HRD (PHX) KMCF KMCF

Flight ID: 20121026H1

Sensor or system						Number or Name

Altitude							AltGPS.1 (RINU)

Accelerometer						AccZfilterI-GPS.1

Dew Point Probe						TDM.2X

Dynamic Pressure						PQM.2

Inertial Selected						INE1

Static Pressure						PSM.2

Temperature Probe				                     TTM.1

Constants File                                                           AAMPSConfig/core/n43.xml

Flight Directory						acdata/2012/MET/20121026H1

Local Met Data			Takeoff (0756z)		Landing (1544z)

Aircraft Static Pressure		  1007.2 mb			     1007.5 mb

Tower Pressure (corrected)	  1007.1 mb			     1007.4 mb

__________________________________________________________________

Notes:

The storm was flown at 8,000 feet pressure altitude.

TDM.2 (Edgetech), the reference dew point sensor, ran away in a band of heavy precip during the first inbound leg northwest of the center from 09:12:00z to 09:14:40z and was patched using statistical methods. TDM.1 (the Buck) also ran away in that same band. The Maycomm (TDM.3) spiked badly in areas of heavy precipitation and was noisy in any areas of precip.

Vertical Winds from 8:49z to 15:09z showed a mean UWZ of -0.01 m/s. SPECIAL NOTE: The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm. It is recommended that these values be used for vertical wind analysis.

All other flight level instruments worked optimally during the flight.

- 23 dropsondes deployed, 21good and transmitted (1 FF + AVAPS power lost as #1 fell)

- 18 externally loaded AXBT were deployed (1 was bad)

Flight Directors: Richard Henning (813) 828-3310 ext. 3086 and A Barry Damiano