[image: image1.png]



[image: image2.png]



N43RF ERROR SUMMARY

Hurricane Sandy 26 Oct 2012 

TDR


Flight ID: 20121026H2
Sensor or system





Number or Name

Inertial Selected (for wind derivation)


INE 1 


Accelerometer






AccZfilterI-GPS.1
Temperature Probe





TTM.1

Dew Point Probe





TDM.2X
Static Pressure






PSF.2 
Dynamic Pressure





PQM.2
Altitude (for vertical wind)




AltI-GPS.1

Flight Directory

acdata/MET/2012/20121026H2
Constants File
              
AAMPSConfig/core/n43.xml

Local Met Data:


Takeoff (2017Z) KMCF
Landing (0355Z) KMCF
Aircraft Static Pressure

1004.5  mb


1003.5 mb

Tower Pressure (corrected)

1004.0 mb


1004.9 mb
Notes:
Takeoff/Landing data:  Data during landing and takeoff are suspect.  It is recommended that ground data not be used for scientific analysis.

SPECIAL NOTE:  The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

During heavy precipitation, TDM.2 (Buck) greatly exceeded ambient temperature values.  As a result, RH values significantly rose above 100%.  During these instances, TDM.2 was modified by direction substitution with values recorded by TDM.1 (Edgetech) using the equation TDM.2X = TDM.1 at the following time intervals.

22:20:57Z - 22:21:11Z
All other AOC instruments utilized in deriving higher order parameters performed optimally.

There were 21 GPS dropsondes and 18 AXBTs deployed from the aircraft.
0 GPS dropsondes or AXBTs failed. 
Flight Director:


Ian Sears

Phone #:


(813) 828-3310 ext. 3039

Page 1 of 1

