NOAA WP-3D N42RF ERROR SUMMARY

1

HURRICANE SANDY 2012

27 Oct 2012 1418A SANDY

HRD (PHX) KMCF KMCF

Flight ID: 20121027H1

Sensor or system						Number or Name

Altitude							AltGPS.1 (RINU)

Accelerometer						AccZfilterI-GPS.1

Dew Point Probe						TDM.2X

Dynamic Pressure						PQM.2

Inertial Selected						INE1

Static Pressure						PSM.2

Temperature Probe				                     TTM.1

Constants File                                                           AAMPSConfig/core/n43.xml

Flight Directory						acdata/2012/MET/20121027H1

Local Met Data			Takeoff (0759z)		Landing (1553z)

Aircraft Static Pressure		  1003.0 mb			     1005.1 mb

Tower Pressure (corrected)	  1003.2 mb			     1005.6 mb

__________________________________________________________________

Notes:

The storm was flown at 12,000 feet PA. Leaving storm at 15:04:30z, descent to 10,000 ft after encountering graupel near freezing level in heavy precip NW quad.

The Maycomm (TDM.3) dew point sensor spiked in areas of heavy precipitation then dropped out to a sensor limit value of -60.4C from 11:37:16z to 15:31:14z then stair-stepped to -25.0C until 15:32:22z before returning to functionality.  TDM.2 (Edgetech), the reference, spiked just after takeoff from 8:11:26z to 8:15:40z. TDM.1 (Buck) values were substituted during this span

Vertical Winds from 8:47z to 15:16z showed a mean UWZ of +0.06 m/s. SPECIAL NOTE: The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm. It is recommended that these values be used for vertical wind analysis.

All other flight level instruments worked optimally during the flight.

- 27 dropsondes deployed, 25 good and transmitted (#25 FF + #3 died at 850mb)

- 18 externally loaded AXBT were deployed (1 was bad)

Flight Director: Richard Henning (813) 828-3310 ext. 3086 and A Barry Damiano