Flight ID: 20121027H2

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.1

Dew Point Probe						TDM.2

Static Pressure							PSM.2

Dynamic Pressure						PQM.2

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2012/20121027H2

Constants File	              20121027H2/AAMPSConfig/core/n42.xml

Local Met Data:			Takeoff (2000Z)		Landing (0316Z)

Aircraft Static Pressure (PSM.2)	1003.9 mb			1006.3 mb

Tower Pressure (corrected)		1004.1 mb			1005.6 mb

Notes:

There were no data gaps.

The Edgetech dewpoint, TDM.2, performed best and was used as default.   TDM.3, the Maycomm TDL, was unusable for the flight.  Dew point values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.

The Novatel Alt, Lat and Lon (GPS.3) had 4 data spikes at 23:13:38Z, 00:28:36Z, 01:28:15Z and 02:02:23Z.  The blended inertial-GPS solution Alt, Lat and Lon (I-GPS.1) is the default position source.

SPECIAL NOTE!!!    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds, vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 21 GPS dropsondes (21 sent) and 18 AXBT’s released from the aircraft.  There were 4 hurricane penetrations.