NOAA WP-3D N42RF ERROR SUMMARY

1

HURRICANE SANDY 2012

28 Oct 2012 1918A SANDY

HRD (PHX) KMCF KMCF

Flight ID: 20121028H1

Sensor or system						Number or Name

Altitude							AltGPS.1 (RINU)

Accelerometer						AccZfilterI-GPS.1

Dew Point Probe						TDM.2X

Dynamic Pressure						PQM.2

Inertial Selected						INE1

Static Pressure						PSM.2

Temperature Probe				                     TTM.1

Constants File                                                           AAMPSConfig/core/n43.xml

Flight Directory						acdata/2012/MET/20121028H1

Local Met Data			Takeoff (0758z)		Landing (1507z)

Aircraft Static Pressure		  1006.0 mb			     1008.2 mb

Tower Pressure (corrected)	  1006.0 mb			     1008.5 mb

__________________________________________________________________

Notes:

The storm was initially flown at 12,000 feet PA. At 10:45:35z, descent to 11,000 ft after encountering graupel near freezing level in heavy precip at N 100NM turn point.

Maycomm (TDM.3) dew point sensor flat lined at sensor limit value of -60.47C for the entire mission.  TDM.2 (Edgetech), the reference, departed from normal functionality during the enroute cruise after leaving the storm from 14:00z to 14:40z and was substituted with TDM.1

Vertical Winds from 8:12z to 14:43z showed a mean UWZ of +0.01 m/s. SPECIAL NOTE: The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm. It is recommended that these values be used for vertical wind analysis.

All other flight level instruments worked optimally during the flight.

- 16 dropsondes deployed, 15 good and transmitted (#3 at 1003z no launch detect )

- 18 externally loaded AXBT were deployed (11 good, 7 were bad)

Flight Directors: Richard Henning (813) 828-3310 ext. 3086 and A Barry Damiano