NOAA WP-3D N42RF ERROR SUMMARY

1

OCEAN WINDS 2013 SHAKEDOWN

11 Jan 2013 WXWXA TRAIN

(PMM) KMCF  KMCF

Flight ID: 20130111H1

Sensor or system						Number or Name

Altitude							AltIGPS.1 (RINU)

Accelerometer						AccZfilterI-GPS.1

Dew Point Probe						TDM.2X

Dynamic Pressure						PQM.2

Inertial Selected						INE1

Static Pressure						PSM.2

Temperature Probe				                     TTM.1

Constants File                                                           AAMPSConfig/core/n43.xml

Flight Directory						acdata/2013/MET/20130111H1

Local Met Data			Takeoff (1334z)		Landing (1524z)

Aircraft Static Pressure		  1024.6 mb			     1025.4 mb

Tower Pressure (corrected)	  1023.6 mb			     1023.8 mb

__________________________________________________________________

Notes:

Flight to Buoy 42003 198 NM SW of MacDill. Descent to 7000 ft Radar Alt at Buoy

Maycomm (TDM.3) dew point sensor was essentially unusable. TDM.2 (Edgetech), the reference, departed from 13:40:41z – 13:59:50z and was substituted by TDM.1 (Buck).

All other flight level instruments worked optimally during the flight.

Vertical Winds from 14:34z to 19:59z showed a mean UWZ of +0.12 m/s. SPECIAL NOTE: The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm. It is recommended that these values be used for vertical wind analysis.

- 2 dropsondes deployed, both were good but not transmitted

- 1 internally loaded AXBT was deployed and was good

Flight Directors: Richard Henning (813) 828-3310 ext. 3086 and A Barry Damiano