NOAA WP-3D N42RF ERROR SUMMARY

1

OCEAN WINDS WINTER 2013

8 Feb 2013 PROJECT FLIGHT #7

NESDIS (PND) CYHZ  CYHZ

Flight ID: 20130208H1

Sensor or system						Number or Name

Altitude							AltIGPS.1 (RINU)

Accelerometer						AccZfilterI-GPS.1

Dew Point Probe						TDM.2X

Dynamic Pressure						PQM.2

Inertial Selected						INE1

Static Pressure						PSM.2

Temperature Probe				                     TTM.1

Constants File                                                           AAMPSConfig/core/n43.xml

Flight Directory						acdata/MET/20130208H1

Local Met Data			Takeoff (1204z)		Landing (2022z)

Aircraft Static Pressure		  1016.4 mb			     1015.8 mb

Tower Pressure (corrected)	  1016.9 mb			     1014.7 mb

__________________________________________________________________

Notes:

Science portion of mission from 1431z – 1731z was flown at 7000 ft radar altitude.

Maycomm (TDM.3) dew point sensor was essentially unusable. TDM.2 (Edgetech), the reference, performed very well throughout the entire mission. TDM.1 (Buck) was slightly noisier than usual during the science portion skimming across the top of the boundary layer.

All other flight level instruments worked optimally during the flight.

Vertical Winds during the science portion showed a mean UWZ of -0.27 m/s (a somewhat larger than normal low bias). The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm. It is recommended that these values be used for vertical wind analysis.

- 14 dropsondes deployed, all were good, none transmitted

- 3 internally loaded AXBT deployed, all were good

Flight Directors: Richard Henning (813) 828-3310 ext. 3086 and A Barry Damiano